# WEBGIS USER AND ADMINISTRATION MANUAL – MEKONG SALT LAB

Proposed Title: WebGIS User and Administration Manual
Vietnamese Title: Sổ tay hướng dẫn sử dụng, cập nhật dữ liệu và quản trị WebGIS Mekong Salt Lab

Version: 1.0 | Updated: 25/07/2026
System: MekongSaltLab – Geospatial Data Monitoring & Visualization Platform for the Mekong Delta

## TABLE OF CONTENTS

- Introduction

- Part A. User Guide

  - A.1 Accessing the Portal

  - A.2 Login / Registration

  - A.3 Exploring the WebGIS Map

  - A.4 Turning Layers On and Off

  - A.5 Viewing the Legend

  - A.6 Inspecting Map Features (Inspector)

  - A.7 Viewing Charts

  - A.8 Filtering Data

  - A.9 Downloading Data

  - A.10 Printing or Exporting the Map

  - A.11 Viewing Con Chim and Hoa Loi Data

  - A.12 Viewing News & Articles

- Part B. Administration Guide

  - B.1 Admin Login

  - B.2 Dashboard Overview

  - B.3 Managing S3 Storage

  - B.4 Managing GIS Data

  - B.5 Adding a Dataset

  - B.6 Updating Attributes

  - B.7 Uploading New Monitoring Data

  - B.8 Managing Monitoring Stations

  - B.9 Managing Water Quality Data

  - B.10 Adding Images

  - B.11 Configuring Popups

  - B.12 Configuring the Legend

  - B.13 User Account Management

  - B.14 Managing Articles

  - B.15 Backup

  - B.16 Restoration

  - B.17 Data Fetch & Export

  - B.18 Landuse Computation

  - B.19 CORS and IP Configuration

- Appendices

  - Detailed Permissions Table

  - Access Information

  - Glossary

## INTRODUCTION

MekongSaltLab is a geospatial data monitoring and visualization platform (WebGIS) serving the Mekong Delta region, with a primary focus on Tra Vinh province. The system is designed to support water resource management, hydrometeorology, and environmental management through powerful interactive mapping tools.

This manual is divided into two parts:

- Part A – User Guide: For all users (USER role) who want to explore and use the system.

- Part B – Administration Guide: For data managers (DATA_MANAGER) and system administrators (ADMIN) who need to manage, update data, and configure the system.

## Part A. USER GUIDE

### A.1 Accessing the Portal

The MekongSaltLab system can be accessed at the following addresses:

| Information | Value |
| --- | --- |
| Frontend URL | https://mekongsaltlab.org |
| Supported Browsers | Google Chrome, Firefox, Microsoft Edge (latest versions) |
| Supported Devices | Desktop computers, tablets, smartphones |

### A.2 Login / Registration

#### A.2.1 Registering a New Account

| Step | Action |
| --- | --- |
| 1 | On the homepage, click the Login button in the top-right corner of the header. |
| 2 | Switch to the Sign Up tab. |
| 3 | Fill in Username, Email, and Password (minimum 6 characters). |
| 4 | Click Sign Up to complete. The system automatically logs you in and redirects to the homepage. |

#### A.2.2 Logging In

| Step | Action |
| --- | --- |
| 1 | Click the Login button on the header. |
| 2 | The Sign In tab is selected by default. Enter your Username and Password. |
| 3 | Click Sign In. Upon success, the header displays your account name and role. |

#### A.2.3 Logging Out

Click the Sign Out button on the right side of the header.

### A.3 Exploring the WebGIS Map

#### A.3.1 Main Interface

| Area | Position | Function |
| --- | --- | --- |
| Header Bar | Top | Logo, slogan, Login/Sign Out button, Admin button (if authorized) |
| Tab Bar | Below Header | 3 tabs: Data Sets, Additional Criteria, Results |
| Left Sidebar | Left side | Tree view of 8 data categories, layer checkboxes, Apply button |
| Map Panel | Center | OpenLayers map with full zoom/pan controls |
| Map Toolbar | Below map | Layers, Download, Timeline, Time-Lapse, Change base layer |
| Footer | Bottom | Quick links, copyright |

#### A.3.2 List of 8 Data Categories

| Category | Description | Sub-layers | Type |
| --- | --- | --- | --- |
| Landsat Imagery | 8 Landsat satellite bands | Band 1-7, Composite RGB | Raster |
| Administration | Tra Vinh administrative boundaries | Province, Commune, Hamlet | Vector |
| Baseline Environment | Baseline environmental data | Landuse Planning (9 districts), Soil Type, Channel System, Ground Water, Road, Landuse Classification | Raster & Vector |
| Ecology | Ecological data | Biodiversity, NDVI, Habitat, Species, Mangroves | Vector |
| Flooding Modeling | Flood simulation | Flooding Distribution, Flood Depth | Vector |
| Hydrology Environment | Real-time hydrological data | Salinity, Tidal, pH (hourly) | Raster |
| Weather | Ecowitt weather data | Weather stations as markers | Point |
| Water Quality | Water quality data | Surface Water, Ground Water | Point + charts |

### A.4 Turning Layers On and Off

- Expand a category: Click the + button to the left of the category name.

- Select a layer: Check the checkbox next to the layer you want to display.

- Choose display type: For layers that support both Raster (R) and Vector (V), click the corresponding button.

- Apply: Click the Apply button at the bottom of the sidebar.

To turn off a layer, uncheck the checkbox and click Apply again.

### A.5 Viewing the Legend

Click the Layers button on the map toolbar to open the legend panel. This panel displays:

- List of active layers currently shown on the map

- Colors and symbols corresponding to each layer

- Ability to toggle individual layers on/off directly from the legend

### A.6 Inspecting Map Features (Inspector)

| Action | Result |
| --- | --- |
| Hover over a data layer | Popup displays detailed information: attributes, pixel values, coordinates |
| Click on a weather station | Popup displays temperature, humidity, wind, rainfall with sparkline charts |
| Click on a water quality station | Popup displays water quality parameters and field photos |

### A.7 Viewing Charts

The system provides charts for the following data:

- Hydrology: Time-series charts for salinity, pH, and tidal levels

- Weather: Sparkline charts for temperature, humidity, wind, and rainfall in station popups

- Water Quality: Comparison charts for parameters across monitoring rounds

- Landuse: Pie/bar charts showing area statistics by land use type and year

### A.8 Filtering Data

Use the Additional Criteria tab next to Data Sets to filter data by:

- Time: Select a time range (day, month, year) via the Timeline

- Parameter: Filter by data type (Salinity, pH, Tidal, etc.)

- Area: Filter by administrative boundary (district, commune)

Filter results are displayed in the Results tab.

### A.9 Downloading Data

#### A.9.1 Downloading Public GIS Data

| Step | Action |
| --- | --- |
| 1 | Click the Download data button on the map toolbar. |
| 2 | Select the data layer you want to download. |
| 3 | Choose the format (GeoJSON, Shapefile, GeoTIFF, etc.). |
| 4 | Click Download. The file is saved to your computer. |

#### A.9.2 Downloading Excel Data

DATA_MANAGER and ADMIN users can export hydrological data to Excel via the Data tab > Export Excel.

### A.10 Printing or Exporting the Map

Currently, users can take a screenshot of the map for printing or sharing. A direct print function will be developed in a future version.

### A.11 Viewing Con Chim and Hoa Loi Data

Con Chim (Con Chim commune, Cang Long district) and Hoa Loi (Hoa Loi commune, Cang Long district) are two case study areas within Tra Vinh province. Related data layers are grouped under the Con Chim and Hoa Loi category in the sidebar.

To view the data:

- In the Data Sets sidebar, expand the Con Chim and Hoa Loi category.

- Select the desired data layers:

  - Land Use layer – cadastral-scale land use maps for both areas.

  - Canals layer – the internal canal network for irrigation.

  - Monitoring Points layer – surface water quality measurement points.

  - Satellite Imagery layer – Landsat imagery covering the areas.

- Click Apply to display on the map.

Tip: Use Zoom to Layer to automatically zoom the map to the exact extent of Con Chim or Hoa Loi.

### A.12 Viewing News & Articles

- Access the News page via the header or footer.

- Articles are displayed as cards with title, excerpt, and publication date.

- Click an article to view its full content.

- You can filter articles by category or search by keyword.

## Part B. ADMINISTRATION GUIDE

### B.1 Admin Login

#### B.1.1 Default Accounts

| Role | Username | Password |
| --- | --- | --- |
| DATA_MANAGER | manager | manager123 |
| ADMIN | admin | admin123 |

Important: Change your password immediately after the first login!

#### B.1.2 Accessing the Admin Panel

- Log in with a DATA_MANAGER or ADMIN account.

- Click the Admin button on the header to access the /data page.

- The admin interface includes the following tabs: Overview, Storage, Data, GIS, Articles, Users (ADMIN only).

### B.2 Dashboard Overview

The Overview tab displays:

- Logged-in account information (username, email, role, creation date)

- System statistics: total users, number of S3 files, storage usage

- Trigger Backup button (ADMIN): manually initiate a system backup

### B.3 Managing S3 Storage

#### B.3.1 S3 Explorer Interface

| Area | Function |
| --- | --- |
| Folder Tree (left) | Browse the S3 directory structure |
| File List (right) | Display files in the selected folder |
| Toolbar | Upload, New Folder, Download, Rename, Copy, Delete, Get Signed URL |

#### B.3.2 S3 Directory Structure

| Root Folder | Description | Format |
| --- | --- | --- |
| gis-data/ | GIS data (raster & vector) | .tif, .geojson, .shp, .kml… |
| station-data/ | Manual station data | .csv |
| monitoring-data/ | Automatic monitoring station data | .csv |
| news-images/ | Article images | .jpg, .png, .webp |

#### B.3.3 Uploading a File

- Browse to the target folder in the folder tree.

- Click Upload > select a file from your computer.

- Enter an S3 Key (optional) to set a different name.

- Enable Overwrite if you want to replace an existing file.

- Click Upload.

#### B.3.4 Creating a New Folder

- Click New Folder.

- Enter the folder name.

- Click Create.

#### B.3.5 Copying and Moving Files

- Copy: Select file > Copy > browse to destination folder > Paste

- Rename: Select file > Rename > enter new name > Save

- Move: Copy + Paste, then Delete the original file

#### B.3.6 Deleting a File

Warning: Deletion is permanent and cannot be undone.

- Select the file > click Delete.

- Confirm in the dialog box.

### B.4 Managing GIS Data

#### B.4.1 GIS Tab Interface

| Area | Function |
| --- | --- |
| Layer List (left) | All GIS layers, displaying ID, name, type (RASTER/VECTOR) |
| Folder Tree (top right) | Folder structure within the selected layer |
| Action Area (bottom right) | New Folder, Upload File, Delete |

#### B.4.2 Managing Folders within a Layer

| Action | Steps |
| --- | --- |
| View folder tree | Select a Layer > the folder tree displays automatically |
| Create new folder | New Folder > enter name > select parent folder > Save |
| Delete folder | Select folder > Delete > confirm (deletes all child files) |

### B.5 Adding a Dataset

A dataset is a data layer within the WebGIS system. To add a new dataset, an ADMIN needs to operate directly via MySQL (there is currently no admin interface for this function). Please contact the system administrator for assistance.

### B.6 Updating Attributes

To update the attributes of a data layer:

- Go to the GIS tab > select the Layer.

- Select the Folder containing the data file.

- Upload the new data file (overwrite the old file if needed).

- The system automatically updates the display on the map.

### B.7 Uploading New Monitoring Data

#### B.7.1 Uploading Manual Station Data (Station Data)

- Go to the Storage tab.

- Browse to station-data/{stationCode}/{parameter}/{year}/{month}/{day}/.

- Click Upload > select the CSV file.

### B.8 Managing Monitoring Stations

#### B.8.1 Viewing the Station List

Go to the Data tab > Manual Stations to view the station list table with the following information: Station ID, Type (surface_water/groundwater), Location, Coordinates (Lat/Lng), Image Code, Status.

#### B.8.2 Adding a New Station

- Click Add Station.

- Enter Station ID, select Type, enter Location, Lat/Lng.

- Enter Image Code (optional).

- Click Save.

#### B.8.3 Importing Stations from Excel

- Click Import Excel.

- Select an Excel file (columns: Station ID, Type, Location, Lat, Lng, Image Code).

- Click Open. The system automatically adds the stations to the database.

#### B.8.4 Editing / Deleting a Station

- Edit: Click on a station > edit form > modify information > Save.

- Delete: Click Delete > confirm (deletes all associated data).

### B.9 Managing Water Quality Data

#### B.9.1 Previewing an Excel File

- Data tab > Water Quality > Preview Excel.

- Select an Excel file > select Sample Date > Preview.

- The system displays a data preview with QCVN standard comparison.

#### B.9.2 Importing Data

- After a successful preview, click Import.

- Enable Overwrite if you want to replace existing data.

- Click Confirm Import.

#### B.9.3 Deleting a Sample

Click Delete next to the sample you want to remove > confirm.

### B.10 Adding Images

#### B.10.1 Station Images

- Upload the image to S3 at station-data/manual-stations/.

- Record the image code (imageCode).

- Update the imageCode for the corresponding station via the Manual Stations tab.

#### B.10.2 Article Images

- Upload the image to S3 at news-images/{article-slug}/.

- When creating/editing an article, enter the image URL in the Image URL field.

### B.11 Configuring Popups

Object information popups are configured on the backend. To change the popup content:

- Edit the configuration file in the backend source code.

- Redeploy the backend.

There is currently no admin interface for configuring popups. This feature will be developed in a future version.

### B.12 Configuring the Legend

The legend is automatically generated based on layer data. To modify it:

- Update the styles/colors in the frontend configuration file.

- Redeploy the frontend.

### B.13 User Account Management

#### B.13.1 Viewing the User List (ADMIN)

Go to the Users tab to view the user table: ID, Username, Email, Role, Enabled, Created At.

#### B.13.2 Adding a New User (ADMIN)

- Click Add User.

- Enter Username, Email, Password.

- Select Role (USER / DATA_MANAGER / ADMIN).

- Enable Enabled to activate immediately.

- Click Save.

#### B.13.3 Editing User Information (ADMIN)

- Click Edit next to the user.

- Modify the information (leave Password blank if not changing).

- Click Save.

Note: ADMIN users cannot change their own role.

#### B.13.4 Disabling / Deleting a User (ADMIN)

- Disable: Turn off Enabled > Save. Can be reactivated at any time.

- Delete: Click Delete > confirm. Cannot be restored.

### B.14 Managing Articles

#### B.14.1 Creating a New Article (ADMIN)

- Go to the Articles tab > New Article.

- Enter Title (slug is auto-generated).

- Select Category: System Update, Data, Announcement, Event, New Feature, Guide.

- Enter Content (supports HTML/rich text).

- Enter Excerpt (2-3 sentence summary).

- Enter Tags (comma-separated).

- Enter Image URL (thumbnail image).

- Enable Featured to mark the article as featured.

- Enable Published to publish immediately (disable = save as draft).

- Click Save.

#### B.14.2 Editing / Deleting an Article (ADMIN)

- Edit: Click Edit > make changes > Save.

- Delete: Click Delete > Confirm.

### B.15 Backup

#### B.15.1 Automatic Backup

| Parameter | Value |
| --- | --- |
| Schedule | Every day at 00:00 |
| Content | Full MySQL database |
| Format | backup/mekong-{yyyyMMdd}_{HHmmss}.sql.gz |
| Storage Location | S3 bucket, prefix backup/ |

#### B.15.2 Manual Backup (ADMIN)

- Go to the Overview tab.

- Click the Trigger Backup button.

- The system performs: database dump > GZip compression > upload to S3.

- Verify the backup file in the Storage tab > backup/ prefix.

### B.16 Restoration

To restore data from a backup file:

- Download the .sql.gz file from S3 (Storage tab > backup/ prefix).

- Decompress: gunzip backup-file.sql.gz.

- Import into MySQL: mysql -u root -p mekong < backup-file.sql.

Warning: This operation is for ADMIN only and must be performed directly on the server.

### B.17 Data Fetch & Export

#### B.17.1 Manual Data Fetch

Ecowitt (Weather Data):

- Data tab > select Ecowitt source.

- Select a device from the dropdown list.

- Select a date > click Fetch Data.

Mekong API (Hydrological Data):

- Data tab > select Mekong source.

- Select a date > click Fetch Data.

#### B.17.2 Exporting Data to Excel

- Click Export Excel in the Data tab.

- Select mode: Monthly or Daily.

- Select Metric (Salinity, pH, WaterLevel, Alkalinity).

- Select Province (Tra Vinh, Ben Tre, Vinh Long).

- Select Time period > click Export.

### B.18 Landuse Computation

- Go to the GIS tab > Landuse Compute.

- View land use area statistics by year.

- Click Compute to rerun the computation (asynchronous).

- Monitor progress via Compute Status.

- Click Inventory to check which raster files have been processed.

### B.19 CORS and IP Configuration

When the server IP address changes:

- Recommended: Run ./manage.sh > select option 9 (Change IP).

- Manual: Edit application.yaml (allowedOrigins) and .env.local (NEXT_PUBLIC_API_URL).

To change a user’s role via MySQL in an emergency:

USE mekong;
-- View all users
SELECT id, username, email, role, enabled FROM users;
-- Change role
UPDATE users SET role = 'DATA_MANAGER' WHERE username = 'user';
-- Disable an account
UPDATE users SET enabled = false WHERE username = 'old_user';

Warning: Back up your data before operating directly on MySQL!

## Appendices

### Detailed Permissions Table

| Function | USER | DATA_MANAGER | ADMIN |
| --- | --- | --- | --- |
| View WebGIS Map | Yes | Yes | Yes |
| View News | Yes | Yes | Yes |
| Download Public Data | Yes | Yes | Yes |
| Login / Registration | Yes | Yes | Yes |
| Dashboard Overview | No | Yes | Yes |
| Upload S3 Files | No | Yes | Yes |
| Delete S3 Files | No | Yes | Yes |
| Create/Copy/Rename S3 Folders | No | Yes | Yes |
| Manage GIS Layers | No | Yes | Yes |
| Manage Monitoring Stations | No | Yes | Yes |
| Import Water Quality Data | No | Yes | Yes |
| Manage Articles (CRUD) | No | No | Yes |
| Trigger Data Fetch | No | Yes | Yes |
| Export Excel | No | Yes | Yes |
| Compute Landuse | No | Yes | Yes |
| Manage Users | No | No | Yes |
| Trigger Backup | No | No | Yes |

### Access Information

| Information | Value |
| --- | --- |
| Frontend URL | https://mekongsaltlab.org |
| Backend API URL | http://103.54.251.212:8084 |
| Swagger API Docs | https://mekongsaltlab.org/swagger-ui/ |

### Default Accounts

| Role | Username | Password |
| --- | --- | --- |
| USER | user | user123 |
| DATA_MANAGER | manager | manager123 |
| ADMIN | admin | admin123 |

Important: Change your password immediately after the first login!

### Glossary

| Term | Explanation |
| --- | --- |
| WebGIS | Web-based Geographic Information System, powered by OpenLayers 10.9 |
| Frontend | User interface (Next.js 15 + React 19) |
| Backend | Server-side processing (Spring Boot 4.0, Java 17) |
| API | Communication between Frontend and Backend (RESTful, JSON) |
| S3 | Centralized file storage (S3-compatible object storage) |
| GeoTIFF | Image file with embedded geospatial coordinates |
| COG | Cloud Optimized GeoTIFF – optimized for web delivery |
| GeoJSON | Vector JSON format for geospatial data |
| DXF | AutoCAD technical drawing format, converted to GeoJSON |
| JWT | JSON Web Token – user authentication |
| UTM 48N | Map coordinate system (EPSG:32648) for the Mekong Delta |
| Layer | A data layer on the map (raster/vector) |
| Raster | Grid-based data (pixels) |
| Vector | Geometric object data (points, lines, polygons) |

Copyright 2026 MekongSaltLab. WebGIS User and Administration Manual – Version 1.0.
Drafted by Hoang and Duy.